How to determine your BMP’s 12-digit hydrologic unit code

Downloading Google Earth and the WATERS data

- Download the free version of Google Earth from http://earth.google.com/intl/en/ and install it on your computer.

- Download the EPA’s WATERS data for Google Earth from http://www.epa.gov/waters/tools/WATERSKMZ/WATERSKMZ.html  and save the .kmz file in a place where you can find it easily.  (The vector file is the preferred file.)

Adding the WATERS file to Google Earth

- Open Google Earth.

- Go to “File” then “open”.

- Navigate to where the WATERS .kmz file was saved, select it, then click “open”.

- Go to “file” then “save” then “save to my places” so the WATERS data will be saved in your Google Earth when you close the program.

Determining your BMP’s 12-digit HUC

- Navigate to your BMP location on the map

- On the left side in the “My Places” area go down to the “MyWATERS” folder and go to the “Surfacewater Features” subfolder

- Check the box next to the “Hydrologic Units” layer. Make sure you give the layer a chance to load.

- After the boundaries have loaded, click on the area in the map where your BMP was installed. A call box should pop up with the name of that subwatershed at the top.  The Code is the 12-digit HUC where your BMP is located.